Consider the following simple network (very similar to example in talk, just with longer and more fiber links):

Use the tables below to develop pricing for a network based on gigabit Ethernet and 10gig Ethernet links for all fiber types (OM1, OM2, OM3, OM4, and OS2).

| Types of Optical Interfaces |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Standard | Speed |  | Supported Fiber |  |  |  | Cost |  |  |  |
| 1000baseSX | 1000Mbs |  | MM |  |  |  | $171 |  |  |  |
| 1000baseLX/LH | 1000Mbs |  | MM or SM |  |  |  | $300 |  |  |  |
| 10GbaseSR | 10Gbs |  | MM |  |  |  | $780 |  |  |  |
| 10GbaseLRM | 10Gbs |  | MM |  |  |  | $930 |  |  |  |
| 10GbaseLR | 10Gbs |  | SM |  |  |  | $3,000 |  |  |  |
| 10GbaseER | 10Gbs |  | SM |  |  |  | $6,360 |  |  |  |
| Distances Supported by Interfaces |  |  |  |  |  |  |  |  |  |  |
| Standard |  | OM1 |  | OM2 | OM3 |  |  | OM4 |  | OS2 |
| 1000baseSX |  | 275m |  | 550m | 1km |  |  | 1.1km |  | No |
| 1000baseLX/LH |  | 500m |  | 500m | 500m |  |  | 500m |  | 10km |
| 10GbaseSR |  | 33m |  | 82m | 300m |  |  | 550m |  | No |
| 10GbaseLRM |  | 220m |  | 220m | 220m |  |  | 220m |  | No |
| 10GbaseLR |  | No |  | No | No |  |  | No |  | 10km |
| 10GbaseER |  | No |  | No | No |  |  | No |  | 40km |

| Cost of Fiber |  |
| --- | --- |
| Fiber Type | Budget Cost per km |
| OM1 | $3,087 |
| OM2 | $2,203 |
| OM3 | $3,739 |
| OM4 | $4,861 |
| OS2 | $2,150 |

Here are some worksheets to use.

What is the total installed fiber length?       3300 m

| Cost of 1 Gigabit Network |  |  |  |  |
| --- | --- | --- | --- | --- |
| Fiber Type | Fiber Cost | Optics Used | Optics Cost | Total Cost |
| OM1 | 3087*3.3= $10,220.10 | Can’t do gigabit on OM1 on links over 275m |  |  |
| OM2 | 2203*3.3= $7,269.90 | Can’t do gigabit on OM2 on links over 550m |  |  |
| OM3 | 3739*3.3= $12,338.70 | 12 x 1000baseSX | 12*171=$2,052 | $1,4390.70 |
| OM4 | 4861*3.3= $16,041.30 | 12 x 1000baseSX | 12*171=$2,052 | $ 18,093.30 |
| OS2 | 2150*3.3= $ 7,095.00 | 12 x 1000baseLX | 12*300=$3,600 | $ 10,695.00 |

| Cost of 10 Gigabit Network |  |  |  |  |
| --- | --- | --- | --- | --- |
| Fiber Type | Fiber Cost | Optics Used | Optics Cost | Total Cost |
| OM1 | 3087*3.3= $10,220.10 | Can’t do gigabit on OM1 on links over 220m |  |  |
| OM2 | 2203*3.3= $7,269.90 | Can’t do gigabit on OM2 on links over 220m |  |  |
| OM3 | 3739*3.3= $12,338.70 | Can’t do gigabit on OM3 on links over 300m |  |  |
| OM4 | 4861*3.3= $6,041.30 | Can’t do gigabit on OM4 on links over 550m |  |  |
| OS2 | 2150*3.3= $ 7,095.00 | 12 x 10GbaseLR | 12*3,000=$36,000 | $43,095.00 |